DEVELOP AN LTE-M COMMUNICATION WORKFLOW FOR WEARABLE MONITORING SYSTEMS.

CODE:

clc;

clear;

close all;

%% LTE-M Wearable Monitoring System

% Simulation Parameters

SNR_dB = 0:2:20;

BER = zeros(length(SNR_dB),1);

PDR = zeros(length(SNR_dB),1);

Reliability = zeros(length(SNR_dB),1);

%% Wearable Sensor Data

HeartRate = randi([60 100],100,1);      % Heart Rate (bpm)

Temperature = 36 + rand(100,1)*2;       % Temperature (36-38°C)

disp('Sample Heart Rate Data');

disp(HeartRate(1:10));

disp('Sample Temperature Data');

disp(Temperature(1:10));

%% Convert Sensor Data to Binary

% Convert temperature to integer (36.5 -> 365)

TempInt = round(Temperature * 10);

% Merge sensor values

sensorData = [HeartRate; TempInt];

% Calculate required number of bits automatically

numBits = ceil(log2(max(sensorData)+1));

% Convert decimal to binary

binaryData = dec2bin(sensorData, numBits) - '0';

% Convert matrix to serial bit stream

txBits = binaryData(:);

%% Simulation

for k = 1:length(SNR_dB)

snr = SNR_dB(k);

%% BPSK Modulation

txSignal = 2*txBits - 1;

%% AWGN Channel

rxSignal = awgn(txSignal, snr, 'measured');

%% Receiver

rxBits = rxSignal > 0;

%% BER Calculation

numErr = sum(txBits ~= rxBits);

BER(k) = numErr / length(txBits);

%% Packet Delivery Ratio

packetSize = 64;

packets = floor(length(txBits)/packetSize);

success = 0;

for p = 1:packets

idx1 = (p-1)*packetSize + 1;

idx2 = p*packetSize;

if isequal(txBits(idx1:idx2), rxBits(idx1:idx2))

success = success + 1;

end

end

PDR(k) = success / packets;

%% Reliability

Reliability(k) = (1 - BER(k)) * 100;

end

%% BER Plot

figure;

semilogy(SNR_dB, BER, '-o','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('BER Performance of LTE-M Wearable Monitoring');

%% PDR Plot

figure;

plot(SNR_dB, PDR,'-s','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Packet Delivery Ratio');

title('Packet Delivery Ratio');

%% Reliability Plot

figure;

plot(SNR_dB, Reliability,'-d','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Reliability (%)');

title('Communication Reliability');

%% Display Results

Results = table(SNR_dB', BER, PDR, Reliability,...

'VariableNames',{'SNR_dB','BER','PDR','Reliability'});

disp('Simulation Results');

disp(Results);

OUTPUT:

Fig.1

Fig.2

Fig.3